# Water Testing

In this activity, you will use the Vernier LabQuests and probes to test the pH of a water sample taken from the pond at the Don Valley Brickworks.

Method:

Part 1:  Collecting a water sample

- Using the container provided, collect a sample of pond water.

Part2:  Preparing the probe

- Remove the storage bottle from the electrode by first unscrewing the lid, then removing the bottle and lid.  Thoroughly rinse the lower section of the probe, especially the bulb, using distilled or de-ionized water.

- Connect the pH Sensor to the lab interface.  The LabQuest should recognize the probe and open the pH file.

- Perform a two-point calibration of the probe by following these steps:

  - Open the 2-point calibration option of the Vernier data-collection program.

  - Rinse the tip of the electrode in distilled water.

  - Place the electrode into the pH 7 buffer solution.  When the voltage reading on the screen stabilizes, enter the corresponding pH value (i.e. 7)

  - Rinse the tip of the electrode in distilled water.

  - Place the electrode into the pH 10 buffer solution.  When the voltage reading on the screen stabilizes, enter the corresponding pH value (i.e. 10)

  - Rinse the electrode with distilled water and replace in the storage container.  It is now ready for use in the sample.

Part 3:  Measuring pH of the water sample

- Remove the pH sensor from the storage bottle.

- Rinse the pH sensor thoroughly with pond water.

- Place the tip of the probe into the sample of pond water (submerge the sensor tip to a depth of 3 -4 cm).

- Click to begin data collection.

- Click to begin a 10 s sampling run.  IMPORTANT:  Leave the probe tip submerged for the 10 seconds that data is being collected.

- When the sampling run is complete, stop data collection and record the average pH.

Part 4:  Clean-up

- Return all samples of pond water to the pond.

- Disconnect the pH sensor from the LabQuest.

- Rinse the pH sensor electrode thoroughly in distilled water and replace in the storage container.  Be sure to fasten the lid securely.

Some things to think about….

Would pH be the same in water samples taken from different depths?

Would the pH be the same if samples were taken from different areas of the pond?

Questions like these could be used in either a teacher directed, coupled inquiry or in an open, fully student initiated inquiry activity.